Loading a single preset into the DSP3-100 and DSP3-700

1. Connect to the DSP3-100/700 and open the DSP3 Series Software.

2. Double-click on the amplifier in the Device List to open the settings.

3. To load the preset settings, go to Load on the top left-hand side of the settings page and navigate to the folder where you saved the preset file. Once there, select the preset file.

*Note that you can only use DSP3-100 files in the DSP3-100 and DSP3-700 files in the DSP3-700; they are not cross compatible.

4. Once you’ve selected the proper preset and select ‘Open’, you will now see the preset settings in front of you. You can confirm the preset you selected by referring to the name on the top bar (See Below).

5. This preset is now loaded onto the amplifier;

6. If you would like to store the preset onto the unit in the event that the preset ever has to be changed and reverted, select ‘Store’ on the top of the settings screen (See Below). This will allow you to overwrite an existing preset or use an open slot on the unit (so be sure to overwrite a preset you will not need on this unit in the future).

6. You are done! The preset is now both loaded as the active preset and saved into the DSP3-100 for future use.